SHOWLINE SL LEDSPOT 300 LUMINAIRE SPECIFICATIONS.

GENERAL.

A.) Overview.

- The luminaire shall be a motorized color mixing luminaire employing a fixed white LED engine.

- The luminaire shall conform to UL 1573 stage and studio use as well as UL 8750 LED standards and tested via ETL to conform to the aforementioned UL specifications. The luminaire shall hold ETL, cETL and CE, and C-Tick markings.

- The luminaire shall conform to USITT DMX-512A(RDM) protocol standards.

- The luminaire shall employ one (1) LED light source engine that will not emit light in the ultra-violet (wavelengths less than 380nm for UV-A,B, or C) or the Infrared spectrum (wavelengths of more than 800 nm).  Units that emit light within this spectrum shall not be accepted.

- The luminaire shall have an integrated control system that provides local controls offering access to set up parameters, reset functions, calibration functions, pre-programmed chase, and status reporting.

- The luminaire shall be a motorized spot with a variable motorized ten (10) to forty-five (45) degree homogenized output.

- The luminaire shall have an output of up to 11500 lumens.

- The luminaire shall have control inputs for:

  - DMX512 with input/output connectivity via a 5 Pin DMX connector

  - RDM with input/output connectivity via a 5 Pin DMX connector

  - DMX512 with input/output connectivity over Ethernet via a RJ45 connector

- All control and power input and output sockets shall be located on the opposite side of the control panel to aid in cable management. There shall be no on/off switch.

B.) Physical

- The construction of the luminaire shall be sheet metal with molded engineering grade plastic in a matt black finish.

- The luminaire shall be of compact dimensions, not exceeding 26.22 inches [666 mm] in height, 17.36 inches [441 mm] in length and 16.53 inches [420 mm] in width.

- The luminaire shall weigh no more than 61.73 lbs. [28 kg].

- The luminaire shall provide mounting capabilities from a pair of quick connect brackets to which approved mounting devices can be attached.  It shall be possible to remove the quick connect brackets without the use of tools. The luminaire shall have four feet constructed from rubber for floor mounting.

- A safety cable attachment point shall be located on the base of the luminaire.

E.) Mechanical Data.

- Variable fans shall be used to provide forced-air cooling for internal components.  There shall be two (2) fan control options.

- A full color LCD menu system shall provide essential system information and operational controls. The LCD display shall automatically orient the display according to the orientation of the unit, thus ensuring the menu is readable in various configurations.

- The finish shall be black.

- The luminaire shall be supplied with a limited two-year warranty when used in normal applications.

- The luminaire shall have a motorized pan and tilt system comprising a pair of two phase stepper motors. The luminaire shall have a pan range of 540 degrees, a tilt range of 260 degrees, and 0.014 degree resolution per step.

- The luminaire shall have a motorized zoom system .The beam angle range shall be from ten (10) to forty-five (45) degrees. It shall be possible to focus the beam to a hard edge throughout the total beam angle range.

- The luminaire shall have a color mixing system comprising of Cyan, Magenta and Yellow coated dichroic glass flags.

- The luminaire shall have a separate color wheel consisting of seven (7) colors + open. It shall be possible to select a color wheel position between two (2) colors to create a split color effect.

- The luminaire shall have a separate CTO (3200K) flag. It shall be possible to operate this independently of the color mixing system and color wheel.

- The luminaire shall have a rotating gobo wheel consisting of six (6) dichroic glass images + open. It shall be possible to rotate each image clockwise or counter clockwise continuously, or alternatively rotate to an indexable fixed position. It shall be possible to shake each image in the beam.

- The luminaire shall have a fixed gobo wheel consisting of nine (9) images + open. It shall be possible to shake each image in the beam.

- The luminaire shall have a three (3) facet prism to split the beam in a circular fashion. It shall be possible to rotate the prism clockwise or counter clockwise continuously, or alternatively rotate to an indexable fixed position.

- The luminaire shall have a separate frost flag.

- The luminaire shall have an iris. It shall be possible to pulse the iris from open to close by selecting a fixed value from the iris control parameter.

- The luminaire shall have a framing shutter system. The framing shutter system shall comprise four (4) blades, each on a separate plane. It shall be possible to control each side of each blade independently. It shall be possible to rotate the framing shutter gate ninety (90) degrees either side of horizontal. It shall be possible to select one of sixteen (16) blade macro effects from a separate blade effect control parameter.

- There shall be separate timing control parameters for the framing shutter system, pan and tilt movement, and the CMY color mixing system.

- The luminaire shall have a separate auto focus control parameter to provide a hard focus at seven and a half (7.5) m throw distance or fifteen (15) m throw distance.

- The luminaire shall have a separate control channel parameter. It shall be possible to individually reset the color system, pan and tilt, gobo wheels, framing shutter system, and prism and frost.

C.) Electrical.

- Supply Voltage shall be 120 to 240V, 50/60Hz. (+/- 10% auto-ranging)

- The luminaire current draw shall not exceed 420 watts.

- The power input and thru connectors shall be the Powercon TrueOne to ensure safe power disconnection while under load.

- The light source shall consist of one (1) two-hundred and seventy (270) watt fixed white LED engine.

- The luminaire shall be ETL and cETL listed, CE and C-Tick marked.

D.) Environmental.

- Maximum operating ambient temperature shall not exceed 104 degrees Fahrenheit (40 degrees Celsius)

- A variable speed cooling system shall be employed to maintain the optimal operating temperature of the luminaire.

- Luminaires shall be low maintenance and environmentally friendly, all units shall be mercury free.

E.) Operation.

- The luminaire shall have control inputs for:

  - DMX512 with input/output via a DMX 5 Pin Male and Female connector

  - RDM with input/output via a DMX 5 Pin Male and Female connector

  - DMX512 with input/output connectivity over Ethernet via a RJ45 connector

Luminaires utilizing proprietary only controls shall not be accepted.

- DMX512 16 bit control shall utilize 53 channels as follows:

            - Pan – High

            - Pan – Low

            - Tilt – High

            - Tilt – Low

            - Intensity – High

            - Intensity – Low

            - Shutter / Strobe

            - Color Wheel

            - Cyan Flag – High

            - Cyan Flag – Low

            - Magenta Flag – High

            - Magenta Flag – Low

            - Yellow Flag – High

            - Yellow Flag – Low

            - CTO Flag

            - Zoom – High

            - Zoom – Low

            - Focus – High

            - Focus – Low

            - Gobo Wheel 1

            - Gobo Wheel 1 – Index/Rotation – High

            - Gobo Wheel 1 – Index/Rotation – Low

            - Gobo Wheel 2

            - Prism

            - Prism – Index/Rotation – High

            - Prism – Index/Rotation – Low

            - Frost

            - Iris

            - Blade 1A Insert – High

            - Blade 1A Insert – Low

            - Blade 1B Insert – High

            - Blade 1B Insert – Low

            - Blade 2A Insert – High

            - Blade 2A Insert – Low

            - Blade 2B Insert – High

            - Blade 2B Insert – Low

            - Blade 3A Insert – High

            - Blade 3A Insert – Low

            - Blade 3B Insert – High

            - Blade 3B Insert – Low

            - Blade 4A Insert – High

            - Blade 4A Insert – Low

            - Blade 4B Insert – High

            - Blade 4B Insert – Low

            - Blade Gate Rotation – High

            - Blade Gate Rotation – Low

            - Blade Macro Effects

            - Blade Speed/Timing

            - Movement Speed/Timing

            - Auto Focus

            - CMY Speed/Timing

            - Dimmer Mode

            - Control Channel

- The luminaire shall include an onboard LCD display and control of the following:

  - Menu settings:

    - Function (Auto-programs and DMX addressing)

    - Information (Fixture status information and software version)

    - Personality (Pan/Tilt, Fan and Display settings, selection of DMX input)

    - Reset (Recalibration options)

    - Effect Adjust (manual selection of fixture parameters)

- Access to the simple ten (10) step auto-progam chase shall be from the control panel of the luminaire. A user definable preset shall store parameter settings for a powerup preset.

- The luminaire shall provide temperature monitoring technology.

  - The current temperature shall be readable in the menu system under Information.

  - It shall be possible to display the temperature information in Celsius or Fahrenheit.

- The luminaire shall include one (1) Fixed White LED engine delivering full field dimming - allowing for both smooth timed fades and fast blackouts.

  - The LED engine used in the luminaire shall be high brightness and proven quality from established and reputable LED manufacturers.

F). DIMMING.

#### 1.) The luminaire shall use 16-bit nonlinear scaling techniques for high-resolution dimming.

#### Five (5) dimming curves shall be selectable via the DMX Dimmer Mode parameter.

#### The luminaire shall be digitally driven using high-speed pulse width modulations (PWM) in concert with power factor control (PFC) to ensure a smooth flicker free dimming curve from 100 to 0 % and shall be imperceptible to video cameras and video related devices.

END OF SPECIFICATION.